A Commercial Motor Vehicle (CMV) pre-trip inspection is one of the most important daily safety tasks a driver performs. It ensures that the vehicle is safe, roadworthy, and compliant with Federal Motor Carrier Safety Regulations (FMCSR §396.13).

Here’s a clear and practical step-by-step checklist of what should be done during a proper pre-trip inspection:

- Perform the inspection before you drive — every day, before leaving your first location.

- Use a systematic approach — go in the same order each time (front to back or top to bottom).

- Have your tools ready: flashlight, gloves, rag, tire gauge, and hammer (for checking tires).

1. Vehicle Overview

Walk around the vehicle and look for:

- Leaks (oil, coolant, fuel, air)

- Leaning or sagging (could indicate suspension or flat tire)

- Obstructions or damage around the vehicle

2. Engine Compartment

Open the hood and inspect:

- Oil level

- Coolant level

- Power steering fluid

- Belts & hoses (no cracks, frays, or leaks)

- Leaks under the engine or radiator

- Alternator & water pump (secure, no play)

- Air compressor (secure, no leaks)

3. Front Axle / Steering

- Steering box & hoses: secure, no leaks

- Pitman arm, drag link, tie rod ends: not loose or worn

- Front suspension: check springs, shocks, and U-bolts

- Tires: proper inflation, tread depth (≥4/32” on steering), no cuts or bulges

- Rims: no cracks, welds, or rust streaks

- Lug nuts: all present, not loose

- Hub oil seal: not leaking, oil at proper level

4. Front Lights and Reflectors

- Headlights (high/low)

- Turn signals, 4-ways

- Clearance and identification lights

- Reflectors and lens covers clean and not broken

5. Driver Side Door Area

- Door opens/closes properly

- Mirror brackets secure

- Fuel tank secure, cap tight, no leaks

- Steps & handholds secure

- Battery box secured and covered

6. Rear of Tractor

- Frame and cross members – no cracks, welds, or bends

- Drive shaft – no play or damage

- Air lines & electrical cord – no leaks, cuts, or dragging

- Fifth wheel – properly greased, locked, and secure

- Mudflaps – secure, not dragging

7. Trailer (if attached)

- Trailer front: air/electrical connections secure

- Landing gear: fully raised, handle secured

- Trailer frame & floor: no holes or bends

- Side panels, doors, hinges, and latches: functional and secure

- Tandem release handle: locked in position

- Trailer suspension and brakes: no leaks, springs secure

- Tires: tread depth ≥2/32”, properly inflated

- Lights & reflectors: all working and clean

- Rear door & seal: close and latch properly

8. Brakes (All Axles)

- Air lines: no leaks or wear

- Brake chambers: secure, not leaking

- Slack adjusters: no more than 1” free play

- Brake pads: not cracked, contaminated, or too thin

(If you have air brakes, do a brake test before driving — including air leakage, warning light/buzzer, and applied pressure test.)

9. In-Cab Inspection

- Seatbelt: works and not frayed

- Horn: working (city & air)

- Windshield & wipers: clear, working, fluid full

- Mirrors: adjusted properly

- Defroster & heater: working

- Gauges: oil pressure, coolant temp, voltmeter, air pressure (builds to 120–140 psi)

- Steering play: no more than 10° (2 inches on a 20” wheel)

- Emergency equipment:

  - Fire extinguisher (charged and secured)

  - 3 reflective triangles

  - Spare fuses (if applicable)

10. Brake System Check (Air Brakes)

- Build air pressure to governor cut-out (120–140 psi)

- Turn off engine, release parking brake, press and hold pedal — shouldn’t lose more than 3 psi per minute (single vehicle)

- Low air warning should activate before 60 psi

- Parking brake pop-out between 20–45 psi

- Rebuild air pressure and test parking brake and trailer brake separately

11. Paperwork

- Vehicle registration

- Insurance card

- Annual inspection sticker/tag

- Previous DVIR (if required) — make sure any previous issues were corrected

FMCSA Reminder

Per 49 CFR §396.13, drivers must:

Be satisfied that the vehicle is in safe operating condition before driving.

If defects are found that could affect safety, report them immediately and don’t drive until repaired.